PSA Oxygen Generator Specification (Vehicle-Mounted Type, 12V, 3LPM Capability)

Product Overview

The DC 12V Vehicle-Mounted PSA Oxygen Generator is a compact and high-efficiency solution designed for mobile applications, including fish transport vehicles, emergency response units, and other on-the-go oxygen supply systems.

This generator delivers 3 LPM of oxygen at 90% purity with an outlet pressure of 1 bar, supplied by a compressor input pressure of less than2 bar. Powered by DC 12V, it ensures reliable and continuous oxygen generation wherever mobility is required.

A key design feature is the separation of the compressor unit and the PSA adsorption bed into distinct modules. This modular configuration:

- Optimizes space utilization within vehicles

- Facilitates easier installation and maintenance

- Enhances airflow and operational efficiency

Using advanced Pressure Swing Adsorption (PSA) technology, nitrogen is selectively adsorbed from compressed air, producing high-purity oxygen consistently. The low-pressure operation ensures safe and reliable performance for mobile applications.

Key Features & Benefits:

- Stable oxygen flow at 3 LPM with 90% ±3% purity

- Modular design separating compressor and PSA bed for optimized space and accessibility

- Low outlet pressure minimizes system stress and enhances safety

- Compact, lightweight, and ideal for vehicle integration

Product Specification

| Category | Specification |
| --- | --- |
| Product Name | Vehicle-Mounted Oxygen Generator (12V) |
| Model Name | O2MOS-12VC2-3LB2 |
| Manufacturer | O2MOS |
| Country of Origin | Republic of Korea |
| Oxygen Purity | 3 LPM @ 90% |
| Oxygen Outlet Pressure | 3 LPM @ 1 bar |
| Operating Voltage | 12VDC (Range: 11VDC–14VDC) For battery or ship power / DC power supply |
| Current Consumption | 20A |
| Compressor | BLDC type |
| Filters Used | Intake HEPA filter, Line filter |
| Control Wiring | Connected via 6-pin connector (Compressor ↔ Bed Part) - 2 wires: DC power from compressor to control PCB - 2 wires: operation switch line - 2 wires: DC fan power supply |
| Control Wiring for control | Connected via 4-pin harness (Bed Part ↔ Driver Console) |
| Oxygen Outlet Port | Ø6 mm |
| Power On/Off Switch | Built-in |
| Power Connection | Anderson connector (for Battery) |
| Oxygen Flow & Pressure Display | Built-in flowmeter and pressure gauge |
| Dimensions / Weight | Bed Part: 350×215×130 mm / 5 kg Compressor Part: 350×215×222 mm / 8 kg |
| Material | Housing: STS304 stainless steel (protrusions included) |

Installation & Wiring Instructions

1) Bed Part

- - Connect the control PCB to the driver console via 4-pin harness

- - Connect compressed air line (10mm urethane tube) to compressor

- - Connect control and power cables via 6-pin connector to compressor

2) Compressor Part

- - Attach anti-vibration pads underneath

- - Fix securely with a dedicated bracket

- - Connect to battery (or power supply) using power connector

Ordering Information

O2MOS-12VC2-3LB2  : Vehicle-Mounted Oxygen Generator (12V)

O2MOS-CARCONTROL-V1 : Driver-Side controller

O2MOS-RO2S-SNL : Emergency Oxygen Alarm Device(Single)