10-Port Oxygen Flow Meter for Live Fish Transport Vehicles

Product Overview

The 10-Port Oxygen Flowmeter is a compact and efficient distribution system that supplies both pure and emergency oxygen from an oxygen generator to each tank in a live fish transport vehicle. Designed at half the size of conventional models, it optimizes installation space while maintaining high performance.

Even in the event of a leak in one tank, the system’s independent channel design minimizes any impact on other tanks, ensuring stable oxygen delivery throughout. Each port is equipped with a 3L fine-adjustable flowmeter, allowing precise control of oxygen flow for optimal live fish health and transport conditions.

Key Features:

- 10 independent oxygen supply ports

- Compact design (approximately 50% smaller than standard units)

- Leak-isolation structure for system stability

- 3L precision flow adjustment for each port

- Ideal for live fish transport and aquaculture applications

Product Specification

| Category | Specification |
| --- | --- |
| Product Name | Flowmeter (10 Ports) |
| Model Name | O2MOS-3L-10LINE-FM |
| Component | Main body, 10 × 3L Flowmeters, 4bar Gauge, 10-Port Manifold, Fittings |
| Flow Rate Display | 0 ~ 3 LPM |
| Oxygen Inlet Line | Ø6 mm One-touch fitting (attached outside box) |
| Oxygen Outlet Line | Ø0.8 mm PVC nipple connection |
| Dimensions / Weight | 600 × 120 × 200 mm / 5 kg |
| Material | STS304 |

Installation & Wiring Instructions

L-shaped body with multiple screw holes at the bottom for secure mounting